Copyright 2019 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

This Product MasterSpec Section is licensed by Deltek, Inc. to Krowne Metal Corporation ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Krowne Metal Corporation, 100 Haul Rd., Wayne, NJ 07470; Phone: (800) 631-0442; Fax: (973) 872-1129; Website: https://krowne.com; Email: sales@krowne.com.

For information about MasterSpec, contact Deltek at (800) 424-5080 or visit masterspec.com.

SECTION 224216.16 - COMMERCIAL SINKS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

MasterSpec includes provisions for LEED 2009, LEED v4, IgCC, and Green Globes. Sustainable design requirements may be inserted in the Section Text using the hypertext links.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Service sinks.

          - Kitchen/utility sinks.

          - Handwash sinks.

          - Sacristy sinks.

          - Shampoo bowls.

          - Manually operated sink faucets.

          - Automatically operated sink faucets.

          - Supply fittings.

          - Waste fittings.

          - Sink supports.

          - Grout.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Retain first subparagraph below for projects that include Section 114000 "Foodservice Equipment" and revise "Kitchen/Utility Sinks" and "Handwash Sinks" articles in this Section correspondingly.

          - Section 114000 "Foodservice Equipment" for NSF-compliant foodservice and handwash sinks.

          - Section 224100 "Residential Plumbing Fixtures" for residential sinks.

      - ACTION SUBMITTALS

Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for sinks.

          - Include rated capacities, operating characteristics[, electrical characteristics,] and furnished specialties and accessories.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

        - Sustainable Design Submittals:

"Product Data" Subparagraph below applies to LEED (including v4), IgCC, ASHRAE 189.1, and Green Globes for fixtures and equipment that consume water.

          - Product Data: For water consumption.

      - INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.

Retain "Coordination Drawings" Paragraph below where coordination is required for installation of products and materials by separate installers. Preparation of coordination drawings requires participation of each trade involved; coordinate with other Sections specifying products and materials to be included. See Section 013100 "Project Management and Coordination."

        - Coordination Drawings: Counter cutout templates for mounting of counter-mounted sinks.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For sinks and faucets to include in operation and maintenance manuals.

Retain subparagraph below if automatic, electronic-sensor faucets are required for Project.

          - In addition to items specified in Section 017823 "Operation and Maintenance Data," include the following:

            - Servicing and adjustments for automatic faucets.

      - MAINTENANCE MATERIAL SUBMITTALS

See Section 017700 "Closeout Procedures" for submission of maintenance material items.

        - Extra Stock Material: Furnish extra materials to Owner that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Faucet Washers and O-Rings: Equal to [10] <Insert number> percent of amount of each type and size installed.

          - Faucet Cartridges and O-Rings: Equal to [5] <Insert number> percent of amount of each type and size installed.

- PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Do not use same lavatory designation as is used for another lavatory in Section 224100 "Residential Plumbing Fixtures," Section 224300 "Healthcare Plumbing Fixtures," and Section 224600 "Security Plumbing Fixtures."

      - SERVICE SINKS

Caution: Sinks in "Service Sinks - Terrazzo, Floor Mounted" Paragraph below require a drainage piping trap under the fixture. Trap is not part of the fixture fittings.

Copy "Service Sinks - Terrazzo, Floor Mounted" Paragraph and re-edit for each type of terrazzo, floor-mounted service sink required.

Insert drawing designation. Use these designations on Drawings to identify each service sink.

        - Service Sinks - Terrazzo, Floor Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Acorn Engineering Company; a Division of Morris Group International.

            - Fiat Products.

            - Florestone Products Co., Inc.

            - Stern-Williams Co., Inc.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Material: Marble chips cast in portland cement to produce a compressive strength of not less than 3000 psi (20.7 MPa), seven days after casting.

            - Shape: [Square] [Rectangular] [Five sided] [Radial front] <Insert shape>.

            - Nominal Size: [24 by 24 inches (610 by 610 mm)] [30 by 30 inches (762 by 762 mm)] [24 by 36 inches (610 by 915 mm)] [32 by 32 inches (815 by 815 mm)] [36 by 36 inches (915 by 915 mm)] <Insert dimensions>.

            - Height: [6 inches (150 mm)] [10 inches (255 mm)] [12 inches (305 mm)] [12 inches (305 mm) with dropped front] <Insert dimensions>.

In "Tiling Flange" Subparagraph below, verify availability if required and indicate side(s) if sink is rectangular.

            - Tiling Flange: [Not required] [On one side] [On two sides] [On three sides].

Indicate front dimension on rectangular sink if rim guard is to be only on one side.

            - Rim Guard: On [front] [all] top surfaces.

            - Color: [Not applicable] <Insert color>.

            - Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

          - Mounting: On floor and flush to wall.

          - Faucet: <Insert service sink drawing designation from "Manually Operated Sink Faucets" Article>.

Caution: Sinks in "Service Sinks - Plastic, Floor Mounted" Paragraph below require a drainage piping trap under the fixture. Trap is not part of the fixture fittings.

Copy "Service Sinks - Plastic, Floor Mounted" Paragraph and re-edit for each type of plastic, floor-mounted service sink required.

Insert drawing designation. Use these designations on Drawings to identify each service sink.

        - Service Sinks - Plastic, Floor Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: CSA B45.5/IAPMO Z124.

            - Material: [Molded polymer] <Insert material>.

            - Nominal Size: [24 by 36 by 10 inches (610 by 915 mm by 255 mm)] [36 by 36 by 10 inches (915 by 915 by 255 mm)] <Insert dimensions>.

Indicate front dimension on rectangular sink if rim guard is to be only on one side.

            - Rim Guard: On [front] [all] top surfaces.

            - Color: [Not applicable] <Insert color>.

            - Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

          - Mounting: On floor and flush to wall.

          - Faucet: <Insert service sink faucet drawing designation from "Manually Operated Sink Faucets" Article>.

Caution: Sinks in "Service Sinks - Molded Stone, Floor Mounted" Paragraph below require a drainage piping trap under the fixture. Trap is not part of the fixture fittings.

Copy "Service Sinks - Molded Stone, Floor Mounted" Paragraph and re-edit for each type of plastic, floor-mounted service sink required.

Insert drawing designation. Use these designations on Drawings to identify each service sink.

        - Service Sinks - Molded Stone, Floor Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Fiat Products.

            - Florestone Products Co., Inc.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.18.2/CSA B125.2.

            - Shape: [Square] [Rectangular] <Insert shape>.

            - Nominal Size: [24 by 24 inches (610 by 610 mm)] [24 by 36 inches (610 by 915 mm)] <Insert dimensions>.

            - Height: [10 inches (255 mm)] <Insert dimensions>.

Indicate front dimension on rectangular sink if rim guard is to be only on one side.

            - Rim Guard: On [front] [all] top surfaces.

            - Color: [Not applicable] <Insert color>.

            - Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

          - Mounting: On floor and flush to wall.

          - Faucet: <Insert service sink faucet drawing designation from "Manually Operated Sink Faucets" Article>.

Copy "Service Sinks - Enameled Cast Iron, Trap Standard Mounted" Paragraph below and re-edit for each type of enameled cast-iron, trap-standard-mounted service sink required.

Insert drawing designation. Use these designations on Drawings to identify each service sink.

        - Service Sinks - Enameled Cast Iron, Trap Standard Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Standard.

            - Commercial Enameling Company.

            - Kohler Co.

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.1/CSA B45.2.

            - Type: Service sink with back.

            - Back: [Two faucet holes] [Plain].

            - Nominal Size: [22 by 18 inches (560 by 457 mm)] [24 by 20 inches (610 by 508 mm)].

            - Color: White.

            - Mounting: [NPS 2 (DN 50)] [NPS 3 (DN 80)] P-trap standard with grid strainer inlet, cleanout, and floor flange.

            - Rim Guard: On front and sides.

          - Faucet: <Insert service sink faucet drawing designation from "Manually Operated Sink Faucets" Article>.

Coordinate "Support" Subparagraph below with "Sink Supports" Article.

          - Support: [Sink carrier] <Insert carrier>.

Caution: Sinks in "Service Sinks - Enameled Cast Iron, Floor Mounted" Paragraph below require a drainage piping trap under the fixture. Trap is not part of the fixture fittings.

Copy "Service Sinks - Enameled Cast Iron, Floor Mounted" Paragraph and re-edit for each type of enameled cast-iron, floor-mounted service sink required.

Insert drawing designation. Use these designations on Drawings to identify each service sink.

        - Service Sinks - Enameled Cast Iron, Floor Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.1/CSA B45.2.

            - Style: With front apron and raised back.

            - Nominal Size: 28 by 28 inches (710 by 710 mm).

            - Color: White.

            - Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

            - Rim Guard: Coated wire.

          - Faucet: <Insert service sink faucet drawing designation from "Manually Operated Sink Faucets" Article>.

        - Service Sinks - Stainless Steel, Floor Mounted: <Insert drawing designation>.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne model [MS-1818] [MS-2424] or comparable product by one of the following:

            - Advance Tabco.

            - Elkay Manufacturing Co.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: NSF 2.

            - Style: Stainless steel with skirt and center drain.

            - Nominal Size: [18 by 18 inches (455 by 455 mm)] [24 by 24 inches (610 by 610 mm)].

            - Material: Stainless steel.

            - Drain: Grid with [NPS 2 (DN 50)] outlet.

            - Rim Guard: None.

In "Faucet" Subparagraph below, insert faucet designation from "Manually Operated Sink Faucets" Article.

          - Faucet: <Insert service sink faucet drawing designation>.

      - KITCHEN/UTILITY SINKS

Copy "Kitchen/Utility Sinks - Stainless Steel, Counter Mounted" Paragraph below and re-edit for each type of stainless steel, counter-mounted sink required.

Insert drawing designation. Use these designations on Drawings to identify each kitchen/utility sink.

        - Kitchen/Utility Sinks - Stainless Steel, Counter Mounted: <Insert drawing designation>.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne; Model [HS-1220] [HS-1225] [HS-1317] [HS-1419] [HS-1425] [HS-1515] [HS-2017] [HS-2619] [HS-3819] or comparable product by one of the following:

            - Advance Tabco.

            - Elkay Manufacturing Co.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: NSF 2.

            - Type: Stainless steel, self-rimming, sound-deadened unit [less ledge back].

            - Number of Compartments: [One] [Two] [Three].

            - Overall Dimensions: <Insert dimensions>.

            - Material: 20 gauge, Type 304 stainless steel.

Retain "Compartment" Subparagraph below for single-compartment sinks.

            - Compartment:

              - Dimensions: <Insert dimensions>.

              - Drain: [1-1/2 inch (38 mm)] [3-1/2 inch (89 mm)] <Insert drain dimension> drain, with basket.

              - Drain Location: [Centered in compartment] <Insert location>.

              - Depth: <Insert dimensions>.

Retain "Each Compartment" Subparagraph below for multiple-compartment sinks.

            - Each Compartment:

              - Dimensions: <Insert dimensions>.

              - Drain: [1-1/2 inch (38 mm)] [3-1/2 inch (89 mm)] <Insert drain dimension> drain, with basket.

              - Drain Location: [Centered in compartment] <Insert location>.

              - Depth: <Insert dimensions>.

          - Faucet(s): <Insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article>.

            - Number Required: One.

            - Mounting: Deck, exposed.

          - Supply Fittings:

            - Standard: ASME A112.18.1/CSA B125.1.

            - Supplies: Chrome-plated brass compression stop with inlet connection matching water-supply piping type and size.

              - Operation: [Loose key] [Wheel handle] <Insert type>.

              - Risers: NPS 1/2 (DN 15), [chrome-plated, rigid-copper pipe] [chrome-plated, soft-copper flexible tube] [ASME A112.18.6/CSA B125.6, braided or corrugated stainless steel flexible hose].

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B125.2.

            - Trap(s):

              - Size: NPS 1-1/2 (DN 40).

              - Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 17-gauge brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 17-gauge brass tube to wall] <Insert trap type>; and chrome-plated brass or steel wall flange.

If retaining "Continuous Waste" Subparagraph below, indicate arrangement on Drawings.

            - Continuous Waste:

              - Size: NPS 1-1/2 (DN 40).

              - Material: Chrome-plated, 17-gauge brass tube.

          - Mounting: On counter with sealant.

Caution: Sinks in "Kitchen/Utility Sinks - Stainless Steel, Freestanding" Paragraph below are unsuitable for commercial foodservice applications unless they have optional NSF 2 certification as a requirement. Freestanding sinks for commercial foodservice applications are generally specified in Section 114000 "Foodservice Equipment." In projects that do not have Section 114000 "Foodservice Equipment" included, paragraph can be used with inclusion of NSF 2 certification requirement.

Copy "Kitchen/Utility Sinks - Stainless Steel, Freestanding" Paragraph and re-edit for each type of stainless steel, freestanding, kitchen/utility sink required.

Insert drawing designation. Use these designations on Drawings to identify each kitchen/utility sink.

        - Kitchen/Utility Sinks - Stainless Steel, Freestanding: <Insert drawing designation>.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne model [18-1C] [18-12DST] [18-18ST] [CS-1836] [CS-1860] [CS-1872] or comparable product by one of the following:

            - Advance Tabco.

            - Eagle Group.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

Retain "Standards" Subparagraph below only for sinks used in commercial foodservice applications.

            - Standards:

              - NSF 2.

            - Type: Stainless steel, freestanding unit with backsplash.

            - Number of Compartments: [One] [Two] [Three].

            - Overall Dimensions: <Insert dimensions>.

            - Material: 20 gauge, Type 300 Series stainless steel.

Retain "Compartment" Subparagraph below for single-compartment sinks.

            - Compartment:

              - Dimensions: <Insert dimensions>.

              - Drain: [1-1/2-inch (38-mm)] [3-1/2-inch (89-mm)] <Insert drain dimension> drain, with basket.

              - Drain Location: [Centered in compartment] <Insert location>.

Retain "Each Compartment" Subparagraph below for multiple-compartment sinks.

            - Each Compartment:

              - Dimensions: <Insert dimensions>.

              - Drain: [1-1/2-inch (38-mm)] [3-1/2-inch (89-mm)] <Insert drain dimension> drain, with basket.

              - Drain Location: [Centered in compartment] <Insert location>.

Retain "Integral Drainboard(s)" Subparagraph below when drainboards are required.

            - Integral Drainboard(s): [None] [Both] side(s).

              - Dimensions Each: <Insert dimensions>.

          - Legs and Feet: Galvanized steel legs with adjustable bullet feet.

In "Faucet" Subparagraph below, insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article.

          - Faucet: Two-handle, mixing-type, pre-rinse unit, <Insert faucet drawing designation>:

            - Number Required: One.

            - Mounting: On backsplash.

          - Supply Fittings:

            - Standard: ASME A112.18.1/CSA B125.1.

            - Supplies: Chrome-plated brass compression stop with inlet connection matching water-supply piping type and size.

              - Operation: [Loose key] [Wheel handle] <Insert type>.

              - Risers: NPS 1/2 (DN 15), [chrome-plated, rigid-copper pipe] [chrome-plated, soft-copper flexible tube] [ASME A112.18.6/CSA B125.6, braided or corrugated stainless steel flexible hose].

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B125.2.

            - Trap(s):

              - Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

              - Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 17-gauge brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 17-gauge brass tube to wall] <Insert trap type>; and chrome-plated brass or steel wall flange.

If retaining "Continuous Waste" Subparagraph below, indicate arrangement on Drawings.

            - Continuous Waste:

              - Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

              - Material: Chrome-plated, 17-gauge brass tube.

      - HANDWASH SINKS

Retain this article only if "Handwash/Hand Sinks" are not specified in Section 114000 "Foodservice Equipment."

Copy "Handwash Sinks - Stainless Steel" Paragraph below and re-edit for each type of stainless steel handwash sink required.

Insert drawing designation. Use these designations on Drawings to identify each handwash sink.

        - Handwash Sinks - Stainless Steel: <Insert drawing designation>.

For 12-inch- (304-mm-) wide hand sink, use Krowne's "Model HS-9L," "Model HS-30L," or "Model HS-35."

For 16-inch- (406-mm-) wide hand sink, use Krowne's "Model HS-2L," "Model HS-11," "Model HS-26L," "Model HS-43," or "Model HS-67."

For ADA handsinks, use Krowne's "Model HS-50," "Model HS-51," or "Model HS-53."

For knee-pedal, use Krowne's "Model HS-15" or "Model HS-41."

For foot pedal, use Krowne's "Model HS-14" or "Model HS-40."

For deck mount, use Krowne's "Model HS-18."

For other variations, use Krowne's "Model HS Series."

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne model [HS-2L] [HS-9L] [HS-11] [HS-14] [HS-15] [HS-18] [HS-26L] [HS-30L] [HS-35] [HS-43] [HS-67] [HS-40] [HS-41] [HS-43] [HS-50] [HS-51] [HS-53] [HS-67] [HS SERIES] or comparable product by one of the following:

            - Advance Tabco.

            - Elkay Manufacturing Co.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standards:

              - NSF 61.

              - NSF 2.

            - Type: [Wall-mounted] [Pedestal-base] stainless steel basin with radius corners, back for faucet, and support brackets.

            - Overall Dimensions: <Insert dimensions>.

Caution: Other gauges may be available from manufacturers. Confirm availability prior to selection.

            - Material: [20 gauge] <Insert gauge>, Type 304 stainless steel.

In "Faucet" Subparagraph below, insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article.

          - Faucet: <Insert faucet drawing designation>:

            - Number Required: One.

            - Mounting: On backsplash.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Retain "Support" Subparagraph below for wall-mounted sinks.

Coordinate "Support" Subparagraph with "Sink Supports" Article.

          - Support: [Wall bracket] <Insert carrier>.

          - Mounting Height: [Standard] [Child] [Accessible in accordance with ICC A117.1].

      - SACRISTY SINKS

Sacristy sinks are custom-made products. Fixtures specified below are typical.

Copy "Sacristy Sinks - Stainless Steel, Wall Hung" Paragraph below and re-edit for each type of stainless steel, wall-hung sacristy sink required.

Insert drawing designation. Use these designations on Drawings to identify each sacristy sink.

        - Sacristy Sinks - Stainless Steel, Wall Hung: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Just Manufacturing.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.3/CSA B45.4.

            - Type: [Single] [Double] bowl.

            - Material: Bowl and hinged cover, 18 gauge, Type 304, 18-8 stainless steel.

            - Nominal Size: Approximately [20 by 23 inches (508 by 584 mm)] [21 by 42 inches (533 by 1070 mm)] <Insert dimensions>.

            - Cover: Hinged with lock on [left] [right] bowl.

          - Faucet: <Insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article>.

            - Location: On [left] [right] bowl.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B125.2.

            - [Left] [Right] Bowl:

              - Drain: Cup with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Drain Piping: NPS 1-1/2 (DN 40) chrome-plated, tubular-brass direct waste without trap, separate waste piping, and wall flange.

            - Opposite Bowl:

              - Drain: Grid with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Trap:

                - Size: NPS 1-1/2 (DN 40).

                - Material: Comply with requirements in "Waste Fittings" Article.

Copy "Sacristy Sinks - Stainless Steel, Two Bowl, Cabinet Mounted" Paragraph below and re-edit for each type of stainless steel, two-bowl, cabinet-mounted sacristy sink required.

Insert drawing designation. Use these designations on Drawings to identify each sacristy sink.

        - Sacristy Sinks - Stainless Steel, Two Bowl, Cabinet Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Just Manufacturing.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.3/CSA B45.4.

            - Type: Cabinet mounted.

            - Material: Bowl and hinged cover, 18-gauge, Type 304, 18-8 stainless steel.

            - Nominal Size: Approximately [21 by 42 inches (533 by 1070 mm)] <Insert dimensions>.

            - Cover: Hinged with lock on [left] [right] bowl.

          - Faucet: <Insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article>.

            - Location: On [left] [right] bowl.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B125.2.

            - [Left] [Right] Bowl:

              - Drain: Cup with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Drain Piping: NPS 1-1/2 (DN 40) chrome-plated, tubular-brass direct waste without trap, separate waste piping, and wall flange.

            - Opposite Bowl:

              - Drain: Grid with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Trap:

                - Size: NPS 1-1/2 (DN 40).

                - Material: Comply with requirements in "Waste Fittings" Article.

Copy "Sacristy Sinks - Stainless Steel, Two Bowl, Counter Mounted" Paragraph below and re-edit for each type of stainless steel, two-bowl, counter-mounted sacristy sink required.

Insert drawing designation. Use these designations on Drawings to identify each sacristy sink.

        - Sacristy Sinks - Stainless Steel, Two Bowl, Counter Mounted: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Just Manufacturing.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sinks from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.3/CSA B45.4.

            - Type: Self-rimming.

            - Material: Bowl and hinged cover, 18 gauge, Type 304, 18-8 stainless steel.

            - Nominal Size: Approximately [17.5 by 37 inches (445 by 940 mm)] [18 by 29 inches (460 by 740 mm)] [22 by 28 inches (560 by 712 mm)] [22 by 33 inches (560 by 0 mm)] [22 by 42 inches (560 by 1070 mm)] <Insert dimensions>.

            - Cover: Hinged with lock on [left] [right] bowl.

          - Faucet: <Insert sink faucet drawing designation from "Manually Operated Sink Faucets" or "Automatically Operated Sink Faucets" Article>.

            - Location: On [left] [right] bowl.

          - Supply Fittings: Comply with requirements in "Supply Fittings" Article.

          - Waste Fittings:

            - Standard: ASME A112.18.2/CSA B125.2.

            - [Left] [Right] Bowl:

              - Drain: Cup with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Drain Piping: NPS 1-1/2 (DN 40) chrome-plated, tubular-brass direct waste without trap, separate waste piping, and wall flange.

            - Opposite Bowl:

              - Drain: Grid with stopper and NPS 1-1/2 (DN 40) tailpiece.

              - Trap:

                - Size: NPS 1-1/2 (DN 40).

                - Material: Comply with requirements in "Waste Fittings" Article.

      - SHAMPOO BOWLS

Caution: Shampoo bowls in this article can be counter mounted, wall mounted using a manufacturer-supplied bracket, or mounted in a base unit. Wall-mounted bowls may require additional underbowl or other support.

Copy "Shampoo Bowl - Enameled Cast Iron" Paragraph below and re-edit for each type of enameled cast-iron shampoo bowl required.

Insert drawing designation. Use these designations on Drawings to identify each shampoo bowl.

        - Shampoo Bowl - Enameled Cast Iron: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Belvedere, LLC.

            - Takara Belmont.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain bowls from single source from single manufacturer.

          - Fixture:

            - Standard: ASME A112.19.2/CSA B45.1.

            - Bowl: Shaped for head rest.

            - Nominal Size: [19 by 19 by 10 inches (483 by 483 by 250 mm)] <Insert dimensions>.

            - Color: [White] <Insert color>.

          - Faucet: Manufacturer's standard with vacuum breaker complying with ASME A112.18.3 and with hose spray head.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article except as follows:

            - Drain: Cup type with hair basket and NPS 1-1/2 (DN 40) tailpiece.

            - Trap Size: NPS 1-1/2 (DN 40).

          - Mounting Material: [Bracket or devices for attaching to counter] [Wall bracket].

Copy "Shampoo Bowl - Plastic" Paragraph below and re-edit for each type of plastic shampoo bowl required.

Insert drawing designation. Use these designations on Drawings to identify each shampoo bowl.

        - Shampoo Bowl - Plastic: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Belvedere, LLC.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain bowls from single source from single manufacturer.

          - Fixture:

            - Standard: CSA B45.5/IAPMO Z124.

            - Material: [Plastic] [FRP] [PMMA] <Insert material>.

            - Bowl: Shaped for head rest.

            - Nominal Size: [19 by 19 by 10 inches (483 by 483 by 250 mm)] <Insert dimensions>.

            - Color: [White] <Insert color>.

            - Mounting Material: [Bracket or devices for attaching to counter] [Wall bracket].

          - Faucet: Manufacturer's standard with vacuum breaker complying with ASME A112.18.3 and with hose spray head.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article except as follows:

            - Drain: Cup type with hair basket and NPS 1-1/2 (DN 40) tailpiece.

            - Trap Size: NPS 1-1/2 (DN 40).

Copy "Shampoo Bowl – Solid-Surface Material" Paragraph below and re-edit for each type of solid-surface-material shampoo bowl required.

Insert drawing designation. Use these designations on Drawings to identify each shampoo bowl.

        - Shampoo Bowl - Solid-Surface Material: <Insert drawing designation>.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Willoughby Industries.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain bowls from single source from single manufacturer.

          - Fixture:

            - Material: Molded, cast polymer.

            - Bowl: Shaped for head rest.

            - Nominal Size: [19 by 19 by 9 inches (483 by 483 by 229 mm)] [22 by 18 by 9 inches (559 by 457 by 229 mm)] <Insert dimensions>.

            - Color: [Gray granite] [Sandstone] [White granite] <Insert color>.

            - Mounting Material: [Bracket or devices for attaching to counter] [Wall bracket].

          - Faucet: Manufacturer's standard with vacuum breaker complying with ASME A112.18.3 and with hose spray head.

          - Waste Fittings: Comply with requirements in "Waste Fittings" Article except as follows:

            - Drain: Cup type with hair basket and NPS 1-1/2 (DN 40) tailpiece.

            - Trap Size: NPS 1-1/2 (DN 40).

      - MANUALLY OPERATED SINK FAUCETS

The U.S. Safe Drinking Water Act (SDWA) has required national compliance with less than or equal to 0.25 percent weighted average lead content at wetted surfaces for pipe, fittings, and devices intended to convey or dispense water for human consumption since January 2014. The IPC and the UPC have the same requirements. Items in compliance with NSF 61 and NSF 372 also comply with this requirement. Some manufacturers choose to meet this requirement through independent testing and have "Certified Lead-Free" products, which may or may not have NSF 61 or NSF 372 certification.

        - Sink faucets intended to convey or dispense water for human consumption are to comply with the U.S. Safe Drinking Water Act (SDWA), with requirements of the Authority Having Jurisdiction (AHJ), and with NSF 61 and NSF 372, or be certified in compliance with NSF 61 and NSF 372 by an ANSI-accredited third-party certification body, in that the weighted average lead content at wetted surfaces is less than or equal to 0.25 percent.

Copy "Commercial Sink Faucets - Manual Type" Paragraph below and re-edit for each manual-type sink faucet required.

Insert designation. Use these designations in sink description to identify each sink faucet.

        - Commercial Sink Faucets - Manual Type: [Single-control, mixing] [Single-control, non-mixing].

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

In "Basis-of-Design Products" Subparagraph below:

Retain Krowne's "Model 12-510L," "Model 12-525L," or "Model 14-520L" model option for single-control mixing, 4-inch (102-mm) center-set lavatory faucet.

Retain Krowne's "Model 12-510L" and "Model 12-525L" options for faucet with brass underbody construction.

Retain Krowne's "Model 14-520L" option for a single-control, mixing, 4 inch center-set faucet.

Retain Krowne's "Model 14-544L" option for a single-control, non-mixing, single hole metering faucet.

Retain Krowne's "Model 16-1 Series" option for a single-hole, remote spout with manual valve, or single hole faucet with manual valve.

Retain Krowne's "Model 16-2 Series" option for a deck-mount, single-hole faucet, with manual valve.

          - Basis-of-Design Product subject to compliance with requirements, provide Krowne [12-510L] [12-525L] [14-520L] [14-544L] [16-1 Series] [16-2 Series] <Insert drawing designation> or comparable product by one of the following:

            - Chicago Faucets

            - T & S Brass

            - <Insert manufacturer's name>.

          - Standard: ASME A112.18.1/CSA B125.1.

          - General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture hole punchings; coordinate outlet with spout and fixture receptor.

Body type in "Body Type" Subparagraph below must match fixture hole punching.

          - Body Type: [4 in. (102 mm) centerset] [Single hole] <Insert type>.

          - Body Material: [Commercial, solid-brass] [Commercial, brass underbody].

          - Finish: [Polished chrome plate] <Insert finish>.

Retain "Maximum Flow Rate" or "Maximum Flow" Subparagraph below.

          - Maximum Flow Rate: [0.2 to 2.2 gpm (0.75 L/min to 8.3 L/min)] <Insert value>.

          - Maximum Flow: [0.37 gal. (0.95 L)] <Insert value> per metering cycle.

          - Mounting Type: [Deck] [Wall], exposed.

          - Valve Handle(s): [Single lever] [Push-button] [Knee pedal] [Foot pedal] <Insert type>.

          - Spout: [Rigid] [Swing] [Swing gooseneck] type.

          - Spout Outlets: [Vandal-resistant] [Laminar flow] [Aerator] <Insert product designation>.

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne [22-5 Series] [22-6 Series] <Insert product designation> or comparable product by one of the following:

              - Sloan Valve.

              - Zurn.

              - <Insert manufacturer's name>.

          - Operation: [Quarter-turn ceramic cartridge], [Manual] [Ceramic cartridge, metering].

          - Drain: [Not part of faucet] <Insert type>.

Copy "Commercial Sink Faucets - Manual Type" Paragraph below and re-edit for each manual-type sink faucet required.

Insert designation. Use these designations in sink description to identify each sink faucet.

        - Commercial Sink Faucets - Manual Type: Two-handle mixing <Insert drawing designation>:

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

In "Basis-of-Design Products" Subparagraph below:

Retain Krowne's "Model 11-4 Series," "Model 14-540L," "Model 14-542L," "Model 14-546L," "Model 15-3 Series," "Model 15-4 Series," and "Model DX-3 Series" for 4-inch center-set, quarter-turn ceramic faucets. "Model 11-4" has brass underbody construction.

Retain Krowne's "Model 14-840L Series" for 8-inch widespread, quarter-turn ceramic faucets.

Retain Krowne's "Model 16-3 Series" and "Model 16-450L" for single-hole, quarter-turn ceramic faucets.

Retain Krowne's "Model 16-480L" for a single-control, non-mixing, 4-inch center-set metering faucet.

Retain Krowne's "Model 10-4 Series" and "Model 14-4 Series" for wall-mount, 4-inch center-set, quarter-turn ceramic faucets. "Model 10-4" has brass underbody construction.

Retain Krowne's "Model 14-8 Series" for wall-mount, 8-inch center-set, quarter-turn ceramic faucets.

Retain Krowne's "Model 15-5 Series" and "Model 15-8 Series" or deck-mount, 8-inch center-set, quarter-turn ceramic faucets.

Retain Krowne's "Model 17-1 Series" and "Model DX-1 Series" for wall-mount, 8-inch center-set, quarter-turn ceramic faucets, and flexible hose with pre-rinse spray.

          - Basis-of-Design Product subject to compliance with requirements, provide Krowne model [10-4 Series] [11-4 Series] [14-4 Series] [14-8 Series] [14-540L] [14-542L] [14-546L] [14-840L] [14-841L] [15-3 Series] [15-4 Series] [15-5 Series] [15-8 Series] [16-480L] [16-3 Series] [16-450L [17-1 Series] [DX-1] [DX-3 Series] <insert drawing designation> or comparable product by one of the following:

            - Chicago Faucets

            - T&S Brass

            - <Insert manufacturer's name>.

          - Standard: ASME A112.18.1/CSA B125.1.

          - General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture hole-punchings; coordinate outlet with spout and fixture receptor.

Body type in "Body Type" Subparagraph below must match fixture hole punching.

          - Body Type: [4-inch (102-mm) centerset] [8-inch (203-mm) widespread] [Single-hole] <Insert type>.

          - Body Material: [Commercial, solid brass] [Brass underbody].

Retain satin finish option only for DX-1 and DX-3 Series. Chrome plate option is not available for these models.

          - Finish: [Polished chrome plate] [Satin]. <Insert finish>.

Retain "Maximum Flow Rate" or "Maximum Flow" Subparagraph below.

          - Maximum Flow Rate: [0.2 to 2.2 gpm (0.75 L/min to 8.3 L/min.)] <Insert value>.

          - Mounting Type: [Deck, exposed] [Deck, concealed] [Back/Wall, exposed].

          - Valve Handle(s): [Wrist blade Lever] [Vandal-resistant wrist blade lever] [Push-button] <Insert type>.

          - Spout: [Rigid] [Rigid, gooseneck] [Swing gooseneck] [Swing] type.

Retain "Pre-Rinse Unit" Subparagraph for applications with kitchen utility sinks that require pre-rinse.

          - Pre-Rinse Unit: Include metal riser and [4-in. (1118 mm)] <Insert length> armored hose with grip handle and [1.15 gpm (4.3 L/min.)] <Insert value> spray head.

          - Spout Outlets: [Vandal-resistant] [Laminar flow] [Aerator] <Insert product designation>.

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne [22-5 Series] [22-6 Series] <Insert product designation> or comparable product by one of the following:

              - Sloan Valve.

              - Zurn.

              - <Insert manufacturer's name>.

          - Operation: [Manual, quarter-turn ceramic] [Ceramic cartridge metering]

          - Drain: [Not part of faucet] <Insert type>.

Copy "Commercial Service Sink Faucets - Manual Type" Paragraph below and re-edit for each manual-type service sink faucet required.

Insert designation. Use these designations in sink description to identify each sink faucet.

        - Commercial Service Sink Faucets - Manual Type: <Insert designation>.

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

In "Basis-of-Design Products" Subparagraph below:

Retain Krowne's "Model 16-127," Model 16-127W," "Model 16-280," and "Model 16-281" for wall-mounted faucets with integral vacuum breaker, wall brace, and quarter-turn cartridges. "Model 16-127W" includes wrist blade handles.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne [16-127] [16-127-W] [16-280] [16-281] or comparable product by one of the following:

            - T & S Brass.

            - Chicago Faucet.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sink faucets from single source from single manufacturer.

          - Description: Wall/back mounted, brass body, with integral service stops, checks, spout with bucket/pail hook, 3/4-inch (20-mm) hose thread end, integral vacuum breaker, inlets 8 in. (203 mm) o.c., and two-handle mixing.

          - Faucet:

            - Standards:

              - ASME A112.18.1/CSA B125.1.

              - ASSE 1001 (VB).

            - Finish: [Rough chrome plated] <Insert finish>.

            - Handles: [Lever] [4-inch (102-mm) wrist blade] <Insert type>.

            - Cartridges: [Quarter-turn ceramic cartridge] <Insert type>.

            - Brace: [Top brace] <Insert type>.

      - AUTOMATICALLY OPERATED SINK FAUCETS

The U.S. Safe Drinking Water Act (SDWA) has required national compliance with less than or equal to 0.25 percent weighted average lead content at wetted surfaces for pipe, fittings, and devices intended to convey or dispense water for human consumption since January 2014. The IPC and the UPC have the same requirements. Items in compliance with NSF 61 and NSF 372 also comply with this requirement. Some manufacturers choose to meet this requirement through independent testing and have "Certified Lead-Free" products, which may or may not have NSF 61 or NSF 372 certification.

        - Sink faucets intended to convey or dispense water for human consumption are to comply with the U.S. Safe Drinking Water Act (SDWA), with requirements of the Authority Having Jurisdiction (AHJ), and with NSF 61 and NSF 372, or be certified in compliance with NSF 61 and NSF 372 by an ANSI-accredited third-party certification body, in that the weighted average lead content at wetted surfaces is less than or equal to 0.25 percent.

Copy "Commercial Sink Faucets - Automatic Type" Paragraph below and re-edit for each automatic-type, electronic-sensor-operated valve required.

Insert designation. Use these designations in sink description to identify required faucet for that sink.

        - Commercial Sink Faucets - Automatic Type: Below-deck electronics, battery powered, electronic sensor-operated, mixing <Insert drawing designation>:

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

In "Basis-of-Design Products" Subparagraph below:

Retain Krowne's "Model 16-196," "Model 16-649," "Model 16-650," and "Model 16-654" for single-hole, deck-mount, battery-operated, automatic-type faucets.

Retain Krowne's "Model 16-675" for single-hole, wall-mount, battery-operated, automatic-type faucets.

Retain Krowne's "Model 16-197" for 4-inch (102-mm) centerset, deck-mount, battery-operated, automatic-type faucets.

Retain Krowne's "Model 16-670" for 4-inch (102-mm) centerset, wall-mount, battery-operated, automatic-type faucets.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne [16-196] [16-197] [16-649] [16-650] [16-654] [16-670] [16-675] <Insert product name or designation> or comparable product by one of the following:

            - Sloan Valve.

            - Zurn.

            - <Insert manufacturer's name>.

          - Standards: ASME A112.18.1/CSA B125.1 and UL 1951.

          - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture hole-punchings; coordinate outlet with spout and fixture receptor.

Body type in "Body Type" Subparagraph below must match fixture hole punching.

          - Body Type: [Single hole] [4 inch (102 mm) centerset] <Insert type>.

          - Body Material: Commercial, solid-brass housing.

          - Finish: [Polished chrome plate] [satin]<Insert finish>.

          - Maximum Flow Rate: [0.35-2.2 gpm (1.3-8.3 L/min.)]. <Insert value>.

          - Mounting Type: [Deck, concealed] [Wall, Concealed] <Insert type>.

          - Spout: [Rigid] [Rigid, gooseneck] [Swing] [Swing gooseneck] type.

          - Spout Outlets: [Vandal-resistant] [Laminar flow] [Aerator] <Insert product designation>.

Retain "Basis-of-Design Products" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

            - Basis-of-Design Product: Subject to compliance with requirements, provide Krowne; Model [22-5 Series] [22-6 Series] <Insert product designation> or comparable product by one of the following:

              - Sloan Valve.

              - Zurn.

              - <Insert manufacturer's name>.

          - Drain: [Not part of faucet] <Insert type>.

      - SUPPLY FITTINGS

        - NSF Standard: Comply with NSF 61 and NSF 372 for supply-fitting materials that will be in contact with potable water.

        - Standard: ASME A112.18.1/CSA B125.1.

        - Supply Piping: Chrome-plated brass pipe or chrome-plated copper tube matching water-supply piping size. Include chrome-plated brass or stainless steel wall flange.

        - Supply Stops: Chrome-plated brass, one-quarter-turn, ball-type or compression valve with inlet connection matching supply piping.

Retain "Wheel handle" option in "Operation" Paragraph below for one-quarter-turn, ball-type stops.

        - Operation: [Loose key] [Wheel handle] <Insert type>.

        - Risers:

          - [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

          - [Chrome-plated, rigid-copper pipe] [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6/CSA B125.6, braided or corrugated stainless steel flexible hose].

      - WASTE FITTINGS

        - Standard: ASME A112.18.2/CSA B125.2.

        - Drain: Grid type with NPS 1-1/2 (DN 40) offset and straight tailpiece.

        - Trap:

          - Size: NPS 1-1/2 (DN 40).

          - Material:

            - Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 17-gauge brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 17-gauge brass tube to wall] [one-piece, cast-brass trap with swivel 17-gauge tubular brass wall bend] <Insert trap type>; and chrome-plated brass or steel wall flange.

      - SINK SUPPORTS

        - Sink Carrier:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Jay R. Smith Mfg Co; a division of Morris Group International.

            - Josam Company.

            - MIFAB, Inc.

            - Wade Drains.

            - WATTS.

            - Zurn Industries, LLC.

            - <Insert manufacturer's name>.

          - Source Limitations: Obtain sink supports from single source from single manufacturer.

          - Standard: ASME A112.6.1M.

      - GROUT

Retain this article if floor-mounted service sinks are included.

        - Standard: ASTM C1107/C1107M, Grade B, post-hardening and volume-adjusting, dry, hydraulic-cement grout.

        - Characteristics: Nonshrink; recommended for interior and exterior applications.

        - Design Mix: 5000 psi (34.5 MPa), 28-day compressive strength.

        - Packaging: Premixed and factory packaged.

- EXECUTION

      - EXAMINATION

        - Examine roughing-in for water-supply piping and sanitary drainage and vent piping systems to verify actual locations of piping connections before sink installation.

        - Examine walls, floors, and counters for suitable conditions where sinks will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install sinks level and plumb in accordance with rough-in drawings.

        - Install supports, affixed to building substrate, for wall-hung sinks.

Indicate on Drawings those sinks that are required to be accessible.

        - Install wall-mounted sinks at accessible mounting height in accordance with ICC A117.1.

        - Set floor-mounted sinks in leveling bed of cement grout.

        - Install water-supply piping with stop on each supply to each sink faucet.

          - Exception: Use ball or gate valves if supply stops are not specified with sink. Comply with valve requirements specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Section 220523.15 "Gate Valves for Plumbing Piping."

          - Install stops in locations where they can be easily reached for operation.

        - Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

        - Seal joints between sinks and counters, floors, and walls using sanitary-type, one-part, mildew-resistant silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

Indicate on Drawings those sinks that are required to be accessible.

        - Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible sinks. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

      - PIPING CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

        - Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

        - Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

      - ELECTRICAL CONNECTIONS

        - Connect wiring in accordance with Section 260519 "Low-Voltage Electrical Power Conductors and Cables."

        - Ground equipment in accordance with Section 260526 "Grounding and Bonding for Electrical Systems."

        - Install electrical devices furnished by manufacturer, but not factory mounted in accordance with NFPA 70 and NECA 1.

        - Install nameplate for each electrical connection, indicating electrical equipment designation and circuit number feeding connection.

Retain one of two subparagraphs below. First subparagraph cross-references Section 260553 "Identification for Electrical Systems" and should be retained for consistent electrical identification. Second subparagraph is an abbreviated version of the product specified in Section 260553 "Identification for Electrical Systems."

          - Nameplate to be laminated acrylic or melamine plastic signs, as specified in Section 260553 "Identification for Electrical Systems."

          - Nameplate to be laminated acrylic or melamine plastic signs with a black background and engraved white letters at least [1/2 inch (13 mm)] <Insert dimension> high.

      - ADJUSTING

        - Operate and adjust sinks and controls. Replace damaged and malfunctioning sinks, fittings, and controls.

        - Install new batteries in battery-powered, electronic-sensor mechanisms.

      - CLEANING AND PROTECTION

        - After completing installation of sinks, inspect and repair damaged finishes.

        - Clean sinks, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

        - Provide protective covering for installed sinks and fittings.

        - Do not allow use of sinks for temporary facilities unless approved in writing by Owner.

END OF SECTION 224216.16